Lab01-运算器及其应用 实验报告

实验过程

32bit ALU

- Datapath & function

- Interface

- 核心

- 电路

- 性能

6bit ALU

- Code

- RTL & 综合仿真

- 仿真

- 性能

- 下载测试

- 输入a

- 输入b（此时f初始为0，加运算）

- 输入f（与运算）

FLS

- Code

- 电路

- 仿真

- 下载测试

总结

本次实验本身难度较低，给同学们提供了一个回忆verilog的机会。再实验过程中回忆、巩固了verilog相关的基本知识。

Appendix

代码

- ALU

module ALU #(

parameter WIDTH = 32    //数据宽度

)(

input [WIDTH-1:0] a, b, //两操作数

input [2:0] f,          //操作功能

output [WIDTH-1:0] y,   //运算结果

output z                //零标志

);

reg [WIDTH-1:0] res;

assign y = res;

assign z = y == 32'b0 ? 1:0;

always @(*) begin

case (f)

3'b000: res = a + b;

3'b001: res = a - b;

3'b010: res = a & b;

3'b011: res = a | b;

3'b100: res = a ^ b;

default: res = 0;

endcase

end

endmodule

- 6bit ALU

module ALU6b(

input [5:0] data,

input en,

input [1:0] enSel,

input clk,

output reg outz,

output reg [5:0] outy

);

reg [5:0] a, b;

reg [2:0] f;

wire [5:0] y;

wire z;

ALU #(6)ALU(.a(a),.b(b),.f(f),.y(y),.z(z));

always @(posedge clk) begin

if(en) begin

case (enSel)

2'b00: a <= data;

2'b01: b <= data;

2'b10: f <= {data[2],data[1],data[0]};

endcase

end

outz <= z;

outy <= y;

end

endmodule

- FLS

module  FLS(

input  clk, rst, btn,

input  [6:0]  d,

output [6:0]  f

);

parameter GET_D0 = 2'b00; // To get the 1st number

parameter GET_D1 = 2'b01; // To get the 2nd number

parameter CAL_F = 2'b10;  // To calculate the next number

wire en;

signal_edge se(clk,btn,en);

reg[1:0] cur_state, next_state;

reg[6:0] f1, f2;

wire[6:0] sum;

ALU #(7)alu(.a(f1),.b(f2),.f(3'b000), .y(sum));

always @(*) begin

if(clk)begin

case(cur_state)

GET_D0: next_state = GET_D1;

GET_D1: next_state = CAL_F;

CAL_F: next_state = CAL_F;

endcase

end

end

always @(posedge clk) begin

if(rst) begin

cur_state <= GET_D0;

f1<=7'b0;

f2<=7'b0;

end

else begin

if(en)begin

case (cur_state)

GET_D0: begin

f1 <= d; f2 <= d;

end

GET_D1: f2 <= d;

CAL_F: begin

f2 <= sum;

f1 <= f2;

end

endcase

cur_state <= next_state;

end

end

end

assign f = f2;

endmodule

// 取时钟上升沿

module signal_edge(

input clk,

input button,

output button_edge

);

reg button_r1,button_r2;

always@(posedge clk)

button_r1 <= button;

always@(posedge clk)

button_r2 <= button_r1;

assign button_edge = button_r1 & (~button_r2);

endmodule

- 6bit ALU 仿真

module ALU6_test();

reg clk,en;

reg [1:0] sel;

reg [5:0] d;

wire [5:0] y;

wire z;

ALU6b ALU6b(.clk(clk),.en(en),.enSel(sel),.data(d),.outy(y),.outz(z));

initial clk=0;

always #5 clk=~clk;

initial

begin

en=1'b0;

#3 en=1'b1;

end

// always #1 en=~en;

initial

begin

sel=2'b00;d=6'b01000;

#100 sel=2'b01;d=6'b000111;

#100 sel=2'b10;d=6'b000000;

#100 d=6'b01;

#100 d=6'b11;

#100 $finish;

end

endmodule

- FLS 仿真

module fls_testbench();

reg[6:0] data;

reg clk,btn,rst;

wire[6:0] out;

fls fls(clk,rst,btn,data,out);

initial clk = 0;

always #1 clk=~clk;

initial

begin

rst = 1; btn = 0;

#5 rst = 0;

#10 data = 7'd2;

#5  btn = 1; #5  btn =0;

#10 data = 7'd3;

#5  btn = 1; #5  btn =0;

#5  btn = 1; #5  btn =0;

#5  btn = 1; #5  btn =0;

#5  btn = 1; #5  btn =0;

#5  btn = 1; #5  btn =0;

#5  btn = 1; #5  btn =0;

#5  btn = 1; #5  btn =0;

#5 $finish;

end

endmodule